Amati Anniversario

SYSTEM:

3 way floorstanding vented box loudspeaker.

CABINET:

Lute shape design, multilayer, constrained-mode damping, enclosure formed using hand selected wood layers, quality graded and oriented for carefully optimized resonances control.

Sub-structural ribs are strategically placed for absolute rejection of spurious vibrations.

TWEETER:

25 mm ultra dynamic linearity ring radiator driver, with dual toroidal wave-guide.

MIDRANGE:

150 mm, ultra dynamic linearity driver. CCAW/Kapton “eddy current free” voice coil.

Dynamically linear magnetic field motor incorporating Kellog and Faraday rings. All moving elements optimally ventilated for “resonance free” response.

Designed synergistically with its vented “acoustic chamber”.

WOOFERS:

220 mm, lightweight Aluminium/Magnesium alloy cone driver in an acoustically amorphous vented chamber. A dual Faraday copper ring long-throw motor system with a 1.5” “eddy current free” voice coil is implemented for an ultra dynamic performance and linearity.

Special coaxial anti-compressor are used, designed to remove cavity resonance and distortions.

CROSS-OVER:

Special multi slope structure with optimized phase/amplitude response. The impedance at low frequencies is controlled for a clear amplifier friendly performance. A unique structured precious metal alloy conductors used, totally coherent on the entire signal path.

Crossing points 350Hz - 4000Hz.

FREQUENCY RESPONSE:

24 Hz – 30.000 Hz, tuning ports included.

SENSIVITY:

92 db SPL (2.83V/1 m)

NOMINAL IMPEDANCE:

4 ohms

POWER HANDLING:

30W – 300W, without clipping.

FINISH:

Graphite or red violin piano gloss.

DIMENSIONS:

265 x 560 x 1170 mm (WxDxH).

WEIGHT:

115 Kg per pair – net weight/140 Kg per pair - shipping weight.